Technical POS

Part I

Product:  BALLAST WATER TREATMENT PLANT

Technical parameter and requirement

  - General

Machinery system and components shall be designed, constructed, installed and supplied in accordance with manufacturer’s standard and in compliance with the requirements of the Classification Society and Regulatory Bodies.

The construction material, design, spare parts and tools for the pumps, heat exchangers, fans, electric components, piping, valves, fittings and accessories built-on or supplied together with main engine and other machinery or equipment shall be in accordance with the relevant manufacturer’s standard.

Final size and output of diesel generating sets, aux. pumps, heat exchangers and miscellaneous will be confirmed at completion of detail design. The preliminary figures indicated herein may be varied according to the detail design.

  - Design conditions

The capacity of machinery and equipment shall be designed based on the following condition unless otherwise specified hereinafter.
     Sea water temperature : 32°C
     Ambient temperature : 45°C
     Atmospheric pressure : 1000 mbar

All machineries to be designed basis ER ambient temp of 45 deg C, and all recommendations of the maker to be followed, especially with respect to installation of control panels (with PCB and controllers etc) in an area in ER where the temp will not exceed 45 degC in peak summer conditions. If required, additional ventilation and / or relocation of such equipment must be considered.

  - Spare parts and tools

Spare parts and tools as required by the classification society and make standards will be supplied and properly installed and secured on board.

Ballast Water Treatment Plant

One (1) set of Electrolyse type ballast water treatment plant of 1000m3/h is mounted in the engine room.

The ballast water treatment shall be approved by IMO and USCG.

All ballast water tanks (including fore peak tank) shall be connected to the BWTS system.

One branch with blind flange for ballast water sampling will be provided on one air pipe of the fore peak tank. For the other ballast water tanks will be sampling taken from the manhole. Sampling point for ballast in way of discharge points from BTWS unit to be arranged as per class requirement.

Two (2) ballast water pumps shall be used for ballasting and de-ballasting. Ballast stripping shall be carried out by two (2) stripping eductor.

| Name | No | Type | Capacity (m3/h x mlc) | Remark |
| --- | --- | --- | --- | --- |
| Ballast pump | 2 | EMD vert. Cent | 500 x 35 | with S.P. |
| Ballast stripping eductor | 2 | Driven by fire water | 50 x 25 |  |

BWTS shutdown is to be activated upon failure of ventilation system.

The cable gland shall be provided when the shipyard approves the drawings.

Part II

| 序号 | 内 容 | 备 注 |
| --- | --- | --- |
| 1 | 根据船级社规则规范及挂旗国要求提供相关证书。 Provide certificates in accordance with Classification Societies, Rules and Regulations, flag authority requirements. |  |
| 2 | 所有密封圈、垫片等涉及橡胶材质的一律不许用再生橡胶。设备商提供的产品中禁止含有石棉成分，并提供无石棉声明和有害物质清单。如果在今后检测中发现有任何石棉成分，供应商（包括代理商和制造商）将承担所有后果。 All gasket rings、gaskets and other rubber things, the reclaimed rubber is not allowed. Asbestos is not allowed in the products supplied by the supplier, and Declaration of Asbestos-free & IHM (Inventory of Hazardous Materials) list should be supplied. If any asbestos is found in future tests, the supplier (including the agent & the manufacturer) shall pay for any aftermath. |  |
| 3 | 技术协议中没有提及到的设备安装必须的安装工具，设备厂家应免费提供。 The installation tools even not mentioned in the TA which is necessary for equipment installation should be supplied by maker for free. |  |
| 4 | 认可图、工作图、完工图等资料的寄送费由设备厂承担。 The equipment factory shall bear the delivery fee for the approval drawings,working drawings and final drawings etc. |  |
| 5 | 设备运输的保护罩应为三防布。 Protection cover(water, fire, oil ,dust proof) should be supplied by engine maker. |  |
| 6 | 排压载时压载泵和扫舱泵要同时运行 ______ main ballast pump and ______ stripping pumps should be parallel operated at the same time during de-ballasting. |  |
| 7 | RS485信号输出/接收至阀门遥控系统 RS485 modbus should be come/to RCV system. |  |
| 8 | 公共报警信号输出至AMS Common alarm signal sends to AMS. |  |
| 9 | 压载泵运行信号输出至压载水处理装置 Ballast pump running signal sends to BWTS. |  |
| 10 | 厂家提供重载询问 The heavy load enquiry should be provided by maker. |  |
| 11 | 软件修改/升级厂家免费提供 The modification and/or update for software should be taken free of charge. |  |
| 12 | 船级社提出的所有关于厂家内部系统修改的要求，厂家需免费修改。 All requirements from the Class for the modification of maker’s inner system should be taken by maker free of charge. |  |
| 13 | 压载水系统需根据最新G8导则要求提供证书，满足D-2要求 BWTS is to be type approved according the new Revised G8 (Res. MEPC.279(70) or the BWMS Code MEPC.300(72)) if installed on ships on or after 28 October 2020, The D-2 requirement must be met. |  |
| 14 | USCG证书和船级社形式认可证书需要厂家提供 USCG certificate and Class type approval certificate should be supplied by maker. |  |
| 15 | 制造厂应提供系泊试验程序以及试航试验程序。 The manufacturer shall provide mooring test procedures and trial trial procedures |  |
| 16 | 滤器必须满足压载泵的最大排量。 The filter must meet the maximum discharge of the ballast pump |  |
| 17 | 制造厂提供的泵，均应配备合适的压力表和真空表。 The pump provided by the manufacturer shall be equipped with the appropriate pressure gauge and vacuum gauge. |  |

Part III

Very Important:

This POS will be a part in the General Requirements, if the manufacturer can’t fulfill some requirements in this POS, please provide a deviation list for the POS and the Technical Agreement, and the reason or the maker’s standard should be mentioned in the deviation list.

If the deviation list is not provided, it’s means that the production will be provided according to this POS, no matter that the Technical Agreement has been marked with any modification or signature. Anything is not mentioned in this POS will according to the Technical Agreement.

After quotation finished, if the maker can meet the requirements in T.A. without deviation, any cost increasing afterwards shall be bear by the maker itself.

Accept by: